Asian School of Management and Technology

(Affiliated to Tribhuvan University)

Full Marks: 60

Time: 3 Hrs.

SET B

Gongabu, Kathmandu

BIM / Third Semester / IT 236: Microprocessor and Computer Architecture

Preboard Examination 2080

Candidates are required to answer the question in their own words as far as practicable.

Group “A”

Brief answer Questions					10×1=10

Attempt All Questions

- Define the terms microprocessor & computer architecture.

- If A= 101101, what is the value of A after perform circular shift Right?

- Define term operand.

- Define Isolated I/O method.

- List the registers of Intel 8085 microprocessor.

- Define associative memory.

- List pipelining hazards

- Define strobe

- What is memory space & address space

- Define general register CPU organization.

Group “B”

Short answer Questions					5×3=15

Attempt Any Five Questions

- Explain about hardware implementation of logic micro operation with its functional table

- Why we needed IOP? Explain

- Explain about hardwired control unit of basic computer.

- List arithmetic instruction of Intel 8085 microprocessor

- Define cache mapping. List the types of cache mapping techniques.

- A non-pipeline system takes 150ns to process a task. The same task can be processed in eight- segment pipeline with clock cycle time of 14ns. Determine the speedup ratio of the pipeline for 1000 tasks.

Group “C”

Long answer Questions					3×5=15

Attempt Any Three Questions

- Why we needed addressing modes? Explain any 5 different types of addressing modes in details.

- Why we need Priority Interrupt? Explain serial priority interrupt method with suitable block diagram.

- Perform -19* +13 by Booth Multiplication Algorithm.

- Write an assembly language program that subtract two 16 bits number & store result in memory locations 2500H

Group “D”

Long answer Questions							2×10=20

Attempt All Questions

- Draw the flowchart of Non-restoring division algorithm. Perform 31/3 by using Non- restoring division algorithm.

- Draw the architecture of Intel 8085 microprocessor & explain each part in details.

******