Block diagram of 8085 microprocessor

Register Array

(yo specifically register array ko lagi ho)

Register in 8085 microprocessor is a sequence of 8 or 16 flipflops that is used to store 8or 16  bits of data. The data stored in it can be directly added, transferred, incremented/decremented or logically combined with the contents of Accumulator. The register array consists of  temp registers(w and z ) ,stack pointer , program counter general registers and increment/decrementor  address latch.

- General purpose registers( B,c,d,e,h,l)

These registers are generally used to store the data given by the user to perform several operations such as addition subtraction etc. The general-purpose registers (B, C, D, E, H, and L) can be used individually as 8-bit registers or in pair (BC, DE, HL) as 16-bit registers.

- Temporary register(W and Z)

These registers are used to store the intermediate results of arithmetic or logical operation performed by ALU. It is also used to temporarily store another operand(other operand than of accumulator) which is then 	passed on to the ALU. It is 8 bit register and are not accessible to the programmer.

- Program Counter(PC)

It is 16 bit register that points the memory location of next instructions to be fetched. When the instruction is fetched the value of PC is incremented by 1 which then point to the next memory location of instruction.

- Stack pointer.

It is 16 bit register that points the memory location of top of stack. The SP is decremented when data is pushed onto the stack and incremented when data is popped from it, as the stack in the 8085 microprocessor follows the LIFO (Last In, First Out) principle.

- Incrementer/Decrementer

This component is responsible to increment the value of the stackpointer by 2 and program counter by 1 when the data is popped out and instruction is fetched respectively. It also decreases the value of stack pointer by 2 when data is pushed on to the stack.

(Yeta samma register array samma explain)

- Accumulator

It is a 8 bit register which is part of ALU. It is used while performing arithmetic and logical operations and is mostly used register in 8085 microprocessor as one of the operands to perform such operations is always stored in it. The results from the operation are also stored in the accumulator.

- Flag register (sir le padako hera)

- Instruction Register(not accessible to programmer)

It is 8 bit register that holds the instruction(operation code), fetched from the memory which is to be executed. It also passes the instruction to the instruction decoder which then converts this instructions so that mp knows what operation to be performed.

- ALU

Arithmetic and logical unit is responsible for performing logical and arithmetic operations. It stores the intermediate result in the temporary register. Similarly it even sets or resets the flag according to the result of the operation as well as stores the final result in accumulator.

- Timing and Control Unit

It provides timing and control signal to the microprocessor to perform operations.

Following are the timing and control signals-

- Control Signals: READY, RD’, WR’, ALE

- Status Signals: S0, S1, IO/M’

- DMA Signals: HOLD, HLDA

- RESET Signals: RESET IN, RESET OUT

# Pin Configuration of 8085

8085 is a 8 bit microprocessor as the data bus used in it is 8 bit.

- Address Bus (A8-A15)

It carries the most significant 8 bits of memory location pointed by stack pointer and program counter. It only carries address and doesn’t deal with data.

- Multiplexed(MUXed) Address/Data bus(AD0-AD7)

It carries the lower 8 bits of memory location pointed by stack pointer or program counter. It is also used to carry data as well. Whether multiplexed address/data bus will be used as address or data bus is determined by ALE(Address Latch Enable).

- Control signals

- ALE(Address Latch Enable)

ALE indicates whether a multiplexed address / data bus will be used as address or data bus. If ALE is set(1) then it will be used as address bus while if ALE is reset it will be used as data bus.

- RD(active low)

Read memory or IO devices indicates data is to be read from the memory location or IO devices and the data bus is ready to accept the data from the memory or IO devices. It reads data when it is active low(0).

- WR(active low)

Write memory or IO devices signal indicates that data is to be written from data bus to memory or IO devices. It writes the data when it is active low(0)

- Status Signal

- IO/M

This signal indicates that whether data is to be read/write from the memory or IO devices. If it is active high (1) read/write operation will be performed in IO devices and if this signal is active low(0) read/write operation will be performed in memory.

- S0 and S1

This signal represents what operation is currently performed on the microprocessor.

Book ko page 20 ko table Rakha yesma.

- Interrupt and Externally Initiated Signals

(Yo Interrupt Service Routine ma ni lekhna milxa)

- TRAP

It is non maskable interrupt and has highest priority among all the interrupt. This interrupt is executed in emergency situations and transfers the control flow  to 0024H location.

- RST 7.5

It is maskable interrupt and has 2nd highest priority among all the interrupts. When this interrupt occurs, the content of Pc is pushed into stack and transfers the control  flow to 003CH address.

- RST 6.5

It is maskable interrupt and has 3nd highest priority among all the interrupts. When this interrupt occurs, the content of Pc is pushed into stack and transfers the control  flow to 0034H address.

- RST 5.5

It is maskable interrupt and has 4th  highest priority among all the interrupts. When this interrupt occurs, the content of Pc is pushed into stack and transfers the control  flow to 002CH address.

- INTR

It is a maskable interrupt and has lowest priority among all the interrupts. It can be disabled by resetting the microprocessor.

- INTA

INTA is  acknowledgment signal generated whenever an INTR interrupt occurs and is accepted . It tells the external device that processor is ready to accept ISR address.

- HOLD

HOLD is a  request signal from DMA controller to gain control of  the address and data bus. It has even more priority than TRAP. When HOLD = 1, the CPU finishes its current operation and releases the buses.

- HLDA

It is HOLD acknowledgement signal which indicates CPU has received the HOLD signal and it will release the address and data bus in next clock pulse. Once the HOLD signal is removed, HLDA automatically returns to 0, and the CPU regains control of the buses.

- RESTIN

This signal resets the microprocessor by setting the PC to zero. It resets the microprocessor when it is active low(0).

- RESETOUT

This signal resets all the peripheral devices connected to the computer. It resets the peripherals when it is active high(1).

- READY

This signal indicates that the peripheral device is ready to send or receive data. If the READY signal is 0 the CPU waits for it to go 1 for read / write operation.

- Serial I/O signals

- SID and SOD

There are two serial IO signals that are used for serial communication.SID is a data line for serial input and SOD is a data line for serial output.

- Power Supply and Clock Frequency

VCC- for 5volts power supply

Vss- for ground

X1 and X2- A crystal is connected at these two pins. The frequency is internally divided by two, therefore, to operate a system at 3MHZ the crystal should have frequency of 6MHZ.

CLK- It is used as the system clock for all the peripherals and devices interfaced with the microprocessor.

# Interrupt

An interrupt in the 8085 microprocessor is a signal that temporarily stops the normal execution of the program to allow the processor to handle a higher-priority task (interrupt service routine, ISR).

## Types of interrupt

- Hardware Interrupt

It is caused by external devices or peripherals. There are 5 interrupt pins i.e TRAP, RST 7.5 ,RST 6.5 RST 5.5 and INTA used as hardware interrupt.

- Software Interrupt

It is a type of interrupt that is caused by executing an  instruction written by programmer in the program. There are 8 software interrupt pins in in 8085 i.e RST0- RST7.

It has less priority than hardware interrupt.

- Vector Interrupt

It is a type of interrupt in which the address of particular Interrupt Serive Routine is known to the processor . For example, TRAP RST7.5,RST6.5 RST5.5

- Non Vector Interrupt

It is a type of interrupt in which the address of Interrupt Service Routine in not known  to the processor and has to be provided by external device to handle the interrupt. For example INTR

- Maskable Interrupt

In this type of interrupt, the interrupt can be ignored or disabled by writing some instructions in the program. For example RST 7.5 – INTR.

- Non Maskable Interrupt

In this type of interrupt, the interrupt cannot be ignored or disabled by writing some instructions in the program. For example TRAP.