Parallel Processing

Parallel Processing is a computing technique in which several operations are performed simultaneously through multiple CPUs.

It divides a single task into various small components which are then processed by  several CPUs concurrently.

This results in faster execution of programs and improves efficiency.

The following figure illustrates an example of a parallel processing.

Book pg 216.

In this figure each unit is independent and can perform sperate functions at a same time. For example a processor can send a data to arithmetic unit for performing calculation and on the same time it can also send data to logical unit for performing logical operation. Thus at the same time both arithmetic and logical operation are executing parallelly.

PIPELINING

Why do we need pipelining?

Pipelining is necessary inorder to maximize the capacity of current processor in without requiring additional circuits or hardware. It uses the concept of parallel processing

Pipelining is a process of arranging the hardware elements of the existing CPU inorder to increase the overall performance.

The instructions in the pipelined processor are simultaneously executed where multiple instructions are overlapped during execution.

Working:

In it the processor is divided into several logical stages/segment where each stage is connected to each other.

Each stage consist of an input register to accept the data and a combinational circuit to perform the operation. The output of the each stage is stored in the interface register which then transfers the result to another stage as input.

The task or instructions within pipeline is divided into multiple successive subtask and each sub task is assigned a segment.

Pipelining is an ongoing, continuous process in which new instructions, or tasks, are added to the pipeline and completed tasks are removed at a specified time after processing completes.

The steps involved in pipelining are as follows:

- Fetch instructions from memory.

- Read the input register, and decode the instruction.

- Execute the instruction.

- Access an operand in data memory.

- Write the result of the operation into the input register of the next segment.

Speed Equation

The speedup equation in pipelining is a measure of how much faster a pipelined architecture can execute instructions compared to a non-pipelined architecture.

. The speedup S is given by:

S=T1/Tp

where:

T1 is the execution time of the program without pipelining (single-stage).

Tp is the execution time of the program with pipelining.

In ideal conditions, the speedup can also be approximated by the number of pipeline stages k if we assume no pipeline stalls and perfect conditions:

S≈k

Note: No. of clock pulse pipeline ma nikalna use this

NO of clock pulse =1 ta task sakauna kati ota clock cycles chaixa + no of segment – 1( k+n-1)

Arithmetic Pipelining

Arithmetic pipelining is computing technique that is used to solve the arithmetic operation related to floating point.

It divided the arithmetic problem into several small problems which are then executed in the pipeline segments.

The flowchart of arithmetic pipelining is for floating addition / subtraction is as follows

In this pipelining , the operation is divided into four segments:

- Compare the exponents by subtraction

To perform the addition or subtraction on floating numbers the exponents of both numbers should be equal. Hence at first the exponents of two numbers are compared and the larger exponent is selected as the exponent of the result.

For example :

X = 0.9504 * 10^3

Y = 0.8200 * 10^2

Difference of exponents: 3-2=1. Thus 3 will be the exponent of the result.

- Align the mantissa

In this segment the mantissa associated with the smaller exponent is shifted right according to the difference received in segment one.

In the above case Y has smaller exponent  and difference obtained is 1.

Hence,

Y= 0.0820 * 10^3

- Add or subtract the mantissa

After the alignment of mantissa both mantissas are either added or subtracted as per the specification .

Here we are performing addition so

Z= X+Y= 1.0324 * 10^3

- Normalize the result

In this segment the result obtained in segment 3 is normalized and the final result is obtained.

Here Z is already normalized so the result is

Z= 1.0324 * 10^3

Instruction Pipeline

Instruction pipelining is a technique used in computer architecture to improve instruction throughput.

It’s execution is just like the queue execution. In queue the data inserted first , will be the data first retrieved. Similarly here also the instruction coming first will be placed in the queue and will be executed in the System.

Multiple instructions are in different stages at the same time, increasing overall efficiency.

It divides each and every instruction into 4 segments they are:

- Fetch Instrction(FI)

This segment retrieves the first instruction from the memory

- Decode Instruction(DA)

The instruction retrieved is decoded in this section and effective address is identified.

- Fetch Operands(FO)

The 3rd section retrieves the necessary operands from the memory

- Execute (EX)

The instructions would finally be executed in the very last segment of a pipeline organization.

After this mention the block diagram and timing of it

Explain the timing from sir ko slide.N

Piple line Hazards

- Data Hazards(Data Dependency)

Data Hazards occurs when one instruction depends on the results of the previous instruction but that results are not available yet.

- ·  Read After Write (RAW) – Occurs when an instruction tries to read a register before a previous instruction writes to it.

- ·  ·  Write After Read (WAR) – Occurs when an instruction writes to a register before an earlier instruction reads from it.

- ·  ·  Write After Write (WAW) – Occurs when two instructions write to the same register out of order.

·

- Structural Hazards(Resource Limitation)

Structural hazards occur when multiple instructions in the pipeline  tries to access the same hardware resource at the same time, but the system lacks enough resources to handle them simultaneously. This results in pipeline stalls (delays).

Iii. Control Hazards(Branch Hazards)

Control Hazards occur when a decoded instruction is found to be a branch instruction. It stops the processing of following instructions until the branch instruction is executed, causing delays in the pipeline.

Controling Pipeline hazards

Hardware Interlocks − Hardware interlocks is a special circuits that detects an instruction which depends on results of the previous instructions.If this happens, the instruction waits (delays execution) until the required data is available.

Operand Forwarding −  Instead of waiting, the processor directly passes the needed data from one instruction to another without delay. This requires extra hardware (like multiplexers) to efficiently route the data within the pipeline.

Branch Prediction

Branch prediction is a guessing technique used by the processor that predicts the outcome of a branch (like an if-else statement) before actually executing it. If the guess is wrong, the incorrect instructions are discarded, but if it's right, time is saved.🚀